# SnowEx Field Atlas: HDF5 storage design

# 1. Goal

Build a per-site HDF5 archive that puts NASA SnowEx LiDAR products (DEM, snow depth, vegetation height) and the matching NASA/JPL UAVSAR L-band radar products (interferometry and amplitude) pixel-aligned on one common 3 m grid per site, as the training and evaluation data for snow-depth models and, later, snow water equivalent (SWE) retrieval. Released as v1.0.05: a base file (product 0.1.0) and an enriched file (enrichment 3.3.0) per site.

# 2. Sites and data inventory

8 sites have overlapping LiDAR + UAVSAR coverage, found by intersecting each LiDAR granule's real footprint polygon with UAVSAR's catalog. A UAVSAR pair counts as a match when it overlaps the footprint and comes within 5 days of the LiDAR survey.

| Site | State | LIDAR product(s) | UAVSAR match | Notes |
| --- | --- | --- | --- | --- |
| Banner Summit | ID | DEM, SD (2 dates), VH (2 dates) | yes, 11 acquisitions | Shares flight lines w/ Dry Creek, Mores Creek |
| Dry Creek | ID | DEM, SD, VH | yes, 2 acquisitions | Shares flight lines w/ Banner Summit |
| Mores Creek | ID | DEM, SD (2 dates), VH (2 dates) | yes, 10 acquisitions | Shares flight lines w/ Banner Summit |
| Fraser | CO | DEM, SD (2 dates), VH (2 dates) | yes, 5 acquisitions |  |
| Cameron Pass | CO | DEM, SD, VH | yes, 5 acquisitions |  |
| Little Cottonwood Cyn | UT | DEM, SD, VH | yes, 6 acquisitions |  |
| Grand Mesa | CO | DEM, SD (6 dates), VH (2 dates) | yes, 27 acquisitions | DEM is a 2016 lidar snow-off DTM (1 m); SD from 5 ASO surveys (2017) + SnowEx 2020 |
| Reynolds Creek | ID | DEM, VH | yes, 1 acquisition | DEM is the 2014 RCEW lidar DEM (1 m); no snow-depth survey is in the archive |

## 2.1 Data volume (final archive)

Sizes of the released v1.0.05 files, in decimal GB. The archive is larger than the downloaded radar products (about 198 GB before clipping) because every layer is stored on the 3 m grid, finer than UAVSAR's native pixels, and Grand Mesa's grid alone holds 286 million cells.

| Site | Base file (.h5) | Enriched file (.enriched.h5) |
| --- | --- | --- |
| Banner Summit | 20.32 GB | 18.40 GB |
| Mores Creek | 4.61 GB | 4.13 GB |
| Fraser | 3.57 GB | 3.25 GB |
| Cameron Pass | 1.30 GB | 1.18 GB |
| Little Cottonwood Cyn | 3.32 GB | 2.87 GB |
| Dry Creek | 1.16 GB | 1.04 GB |
| Grand Mesa | 107.94 GB | 114.03 GB |
| Reynolds Creek | 1.08 GB | 0.95 GB |
| Total (16 files, 289.15 GB) | 143.31 GB | 145.84 GB |

# 3. Pipeline design

## 3.0 What the pipeline does, end to end

| Step | Action |
| --- | --- |
| 1. Plan | Read the frozen product inventory and the LIDAR/UAVSAR match list; group by unique UAVSAR product so sites built in the same run download a shared flight once. |
| 2. Grid + LIDAR | Derive the site's common grid from its reference DEM (3.2); write identification/, resample each DEM/SD/VH product onto the grid (3.4), write it into the site's .h5, then write matches/. |
| 3. Fetch UAVSAR | Download each unique product once, by its recorded product URL (found via footprint search). |
| 4. Reproject + clip | Resample onto the common grid of every site sharing that product (3.2, 3.4), reading only the site's extent plus an 8-pixel margin of the source raster. |
| 5. Write | Stream the clipped, aligned arrays straight into each site's .h5. |
| 6. Clean up | Delete the raw UAVSAR download once every site needing it has been written. |
| 7. Check | --mode verify reopens a file and checks every array's shape, transform and CRS against the site's common grid. |
| 8. Enrich | Write <site>.enriched.h5: clean the layers, regroup radar by acquisition, add slope, aspect, canopy fraction, coherence masks and incidence angles (4). |
| 9. Verify | Re-check the enrichment from the stored values, audit every file independently, and write SHA-256 manifests. |

## 3.1 One download per flight, shared across sites

- Banner Summit, Dry Creek and Mores Creek share UAVSAR flights on Lowman line 23205; Banner Summit and Mores Creek also share line 05208.

- Each unique product is downloaded once, clipped separately for every overlapping site, then the raw file is deleted.

## 3.2 Common grid per site

- Every array in a site's file (LIDAR and UAVSAR alike) shares one CRS, one affine transform, one pixel shape. The CRS and pixel alignment come from the site's reference LiDAR DEM, the pixel size is fixed at 3 m, and the grid extent is where that DEM and the site's science data overlap.

- The same row and column are the same ground location in every layer; the verify and audit steps check every array's shape, transform and CRS against the grid.

Grand Mesa's reference is the 2016 lidar-derived snow-off DTM from the SnowEx HRSI collection (1 m, resampled to 3 m, EPSG:32612).

Reynolds Creek has no snow-depth survey in the archive. Its reference is the 2014 Reynolds Creek Experimental Watershed lidar DEM (1 m, resampled to 3 m, EPSG:26911), so the site carries terrain, vegetation and radar but no snow-depth labels.

## 3.3 Resolution

- LIDAR: QSI and ASO products are published at 3 m; Grand Mesa's DTM and Reynolds Creek's DEM are 1 m and are resampled to 3 m. UAVSAR: 0.00005556° pixels, about 4.5 m × 6.2 m on the ground in Idaho and 4.8 m × 6.2 m in Colorado and Utah (non-square); every file records the nominal 4.45 m × 6.18 m.

- The 3 m LIDAR grid is the common grid; UAVSAR is upsampled onto it, and its native resolution is stored as uavsar_native_resolution_m.

This keeps the LIDAR labels at full resolution; models can average to coarser blocks when training.

## 3.4 Resampling method

- .int (complex interferogram): nearest-neighbor, to avoid averaging across a 2π wrap boundary.

- .unw, .cor, amplitude and all LIDAR variables: bilinear.

## 3.5 Naming, documentation, and NISAR alignment

- In the base file, UAVSAR groups are named by site, source flight line and dates:

BANNERSUMMIT_CUTFROMLOWMAN23205_20200131_20200213

- The enriched file regroups radar by acquisition — UAVSAR/<date1>_<date2>/<flight line>/ — with amplitude and interferometry together, since they come from the same pair of passes; the base group name is kept as original_group_name.

- Original JPL product ID and ASF file ID kept as attributes for traceability.

- Every science array carries a plain-English description attribute plus its grid and source lineage; derived layers also carry units.

- The overall structure (science data under science/, file-level metadata in a separate identification/ group) borrows its group names from NISAR HDF5 products.

# 4. Full HDF5 schema (v1.0.05)

Shown for Banner Summit's enriched file — 2 SD dates, 2 VH dates, DEM, derived layers and 11 UAVSAR acquisitions, on flight lines shared with Dry Creek and Mores Creek. The base file (banner_summit.h5) holds the same LIDAR layers before enrichment cleaning, snow-pit tables under insitu/, and radar by product type (INTERFEROMETRY_GRD/, AMPLITUDE_GRD/, DEM_TIFF/). Italic arrows explain what each item actually is.

banner_summit.enriched.h5

├─ identification/   → file-level metadata, no science data — the “label on the box”

│   common_crs_epsg, common_crs_wkt   → the shared map projection every array in this file uses

│   common_grid_transform, common_grid_shape, common_grid_resolution_m   → the exact 3 m pixel grid (origin, spacing, size) everything is resampled onto

│   reference_product, reference_filename, grid_extent_rule   → which lidar DEM defines the grid, and how its extent was chosen

│   uavsar_native_resolution_m, resampling_note   → UAVSAR's true original pixel size, and how/why it was resampled

│   site_name, site_key, state, product_version, enrichment_version, created_date   → basic identifying info about this file

│

├─ science/   → the measured and derived data, split into LIDAR/ and UAVSAR/

│   ├─ LIDAR/   → data from the airborne laser surveys

│   │   ├─ DEM/   → bare-earth elevation — the ground surface with snow and plants stripped away

│   │   │     grids/elevation   → the actual elevation values (meters), plus the source file and survey date

│   │   ├─ SD/   → snow depth — how deep the snow was, in meters (snow-on survey minus snow-off survey)

│   │   │     20200218/snow_depth,  20210315/snow_depth   → one snow depth map per survey date

│   │   ├─ VH/   → vegetation height — how tall the trees/brush are

│   │   │     20200218/veg_height, 20210315/veg_height   → one vegetation height map per survey date

│   │   └─ DERIVED/   → computed from the layers above

│   │         slope, aspect   → terrain steepness and downhill direction, in degrees

│   │         forest_cover_fraction_20200218, _20210315   → share of nearby cells with vegetation ≥ 2 m

│   │

│   └─ UAVSAR/   → data from the airplane-mounted radar — clipped to this site, resampled onto the LIDAR grid

│         20200213_20200221/LOWMAN23205/   → one acquisition: the Lowman flight line, comparing Feb 13 to Feb 21, 2020

│           attrs: original_product_id, original_group_name, acquisition_dates,   → traceability: JPL's product ID, the base-file group name and the two real flight dates

│                  radar_wavelength_cm, radar_look_direction, platform_altitude_m   → radar and flight geometry from JPL's annotation file

│           HH/  amp1, amp2, int, unw, cor, coherence_mask   → horizontal-send / horizontal-receive antenna combination — 6 arrays (below)

│                • amp1, amp2 = amplitude   → radar brightness on each pass — how much energy bounced back

│                • int = raw interferogram   → the radar phase difference between the passes — a complex number, wraps every 360°

│                • unw = unwrapped phase   → the int value “unrolled” into a continuous number — the usable surface-change signal

│                • cor = coherence   → 0 to 1, how trustworthy the phase measurement is at that pixel (1 = reliable)

│                • coherence_mask   → 1 where coherence ≥ 0.30, 0 below, 255 where missing

│           HV/, VH/, VV/  (same arrays)   → the other send/receive combinations, where the flight recorded them; a few pairs have no unw

│           GEOMETRY/  incidence_angle_flat, local_incidence_angle   → the angle the radar meets the ground, ignoring and including the slope

│

└─ matches/   → a table recording which LIDAR date was paired with which UAVSAR acquisition, how many days apart, and whether it counted as a match

lidar_product_type, lidar_date, lidar_filename, uavsar_level, uavsar_dates, uavsar_flight_id, gap_days, group_name, verdict